Quick Guide for Wake on WLAN

Date: 2014/01/17

Version: 0.1

Quick Start Guide for Wake on Lan

(1) Support list:

.) USB interface: 8188EU, 8188CU, 8192DU, 8192EU, 8812AU.

.) SDIO interface: 8189ES, 8723BS.

(2) Requirements of wakeup via in-band and out-band methods:

.) In-band requirements:

- SDIO Interface:

- SDIO host MUST support remote wakeup feature.

- SDIO data1 MUST be wakeup source in the host platform.

- The platform MUST keep power to WiFi chip in suspend state.

- The platform MSUT work fine between suspend and resume.

- USB Interface:

- USB host MUST support remote wakeup feature.

- The platform MUST keep power to WiFi chip in suspend state.

- The platform MSUT work fine between suspend and resume.

.) Out-band requirements:

- The GPIOof the PLATFORM MUST be wakeup source.

- The platform MUST keep power to WiFi chip in suspend state.

- The platform MSUT work fine between suspend and resume.

- The WIFI module MUST have the GPIO wakeup pin.

(3) Driver Configuration for Wake on Lan:

.) In-band configuration:

If using SDIO DATA1 pin or USB protocol D+/D- toggle in-band method to wakeup the host, driver need to do is only switch CONFIG_WOWLAN from “n” to “y” in Makefile as figure 1.

(figure 1)

.) Out-band configuration:

If using out-band method, driver need to do is modify Makefile and config GPIO. The detail is as following:

- Makefile Configuration:

Switch CONFIG_WOWLAN and CONFIG_GPIO_WAKEUP from “n” to “y” as figure 2.

(figure 2)

- GPIO Configuration:

Please contact with RTK technical support team.

(4) The wake yo reasib table:

The DUT could be waked up by the WIFI chip with the following reasons:

| Reason Value | Description | Note |
| --- | --- | --- |
| 0x01 | Receive pairwise key change packet. |  |
| 0x02 | Receive group key change packet. | 8723BS support GTK offload. |
| 0x04 | Receive disassociate packet. |  |
| 0x08 | Receive de-auth. packet. |  |
| 0x10 | AP power off, or could not receive AP’s beacon in a period time |  |
| 0x21 | Receive magic packet. |  |
| 0x22 | Receive unicast packet. | The unicast packet included IP level. |